Practice 8 Defense

Topology:

Task:

- All routers exchange routes using OSPF (Area 0), including LAN networks

- The 192.168.20.0/28 (Server network) is reachable using static routing only

- Full connectivity is achieved between all devices

Addressing Table:

| Device | Interface | IP address/Prefix | Subnet mask | Default gateway |
| --- | --- | --- | --- | --- |
| R0 | Gig0/0 | 192.168.10.1/24 | 255.255.255.0 | N/A |
|  | Gig0/1 | 10.1.1.1/30 | 255.255.255.252 | N/A |
| R1 | Gig0/0 | 10.1.1.2/30 | 255.255.255.252 | N/A |
|  | Gig0/1 | 10.2.2.1/30 | 255.255.255.252 | N/A |
|  | Gig0/2 | 192.168.20.1/28 | 255.255.255.240 | N/A |
| R2 | Gig0/0 | 10.2.2.2/30 | 255.255.255.252 | N/A |
|  | Gig0/1 | 192.168.30.1/27 | 255.255.255.224 | N/A |
| PC0 | Fa0 | 192.168.10.5/24 | 255.255.255.0 | 192.168.10.1 |
| PC1 | Fa0 | 192.168.10.6/24 | 255.255.255.0 | 192.168.10.1 |
| PC2 | Fa0 | 192.168.30.5/27 | 255.255.255.224 | 192.168.30.1 |
| PC3 | Fa0 | 192.168.30.6/27 | 255.255.255.224 | 192.168.30.1 |
| Server | Fa0 | 192.168.20.5/28 | 255.255.255.240 | 192.168.20.1 |

Step 1: Configure IP addressing:

- Assign the correct IP address and subnet mask to every interface on Router0, Router1, and Router2

- Configure the default gateway on each PC based on the table

Step 2: Configure static routes to the Server on Router0 and Router2

Step 3: Configure OSPF with the following requirements:

- Process ID: 1

- Area: 0

- Manually configure router-id on each router:

| Router | Router ID |
| --- | --- |
| Router0 | 1.1.1.1 |
| Router1 | 2.2.2.2 |
| Router2 | 3.3.3.3 |

Step 4: Verify full end-to-end connectivity:

- Every router should have reachability to all remote networks

- All end devices (PC0, PC1, PC2, PC3, Server) should be able to successfully communicate with each other